# SIH1557 technical mechanism report

## Build-level design for mining-dumper tyre intelligence

## 0. Design decision

Build three explainable intelligence services around one event-driven tyre data model:

- Vision service: image → damage presence/type/location/severity.

- Risk service: telemetry + vision + history → Low/Medium/High failure risk with reasons.

- Hotspot service: GPS/IMU + damage/failure outcomes → exposure-normalized mine-road risk map.

Do not train a claimed time-to-failure model for the hackathon. A failure-time model needs repeated observations linked to the same tyre and a defined event endpoint. Mining studies do provide precedents for proportional-hazard reliability modelling and useful-life prediction, but that does not create the required public longitudinal dataset for this prototype[1][2]. Use fuzzy risk as decision support, not as a calibrated probability of failure.

Commercial systems already cover much of the monitoring perimeter: pressure/temperature, GPS, alerts, analytics, fleet integration and severity maps[3][4]. TOMS also covers inspection images, tyre work, rotations, end-of-life changes, TPMS integration and predictive analytics[5]. The defensible prototype contribution is therefore the transparent fusion mechanism and deployable interfaces, not a claim that each component is new.

## 1. Canonical data model and system contract

Every record must be traceable through:

truck_id → tyre_id → fitment_id → wheel_position → timestamp.

### 1.1 Core entities

Truck
  truck_id, model, payload_capacity_t, mine_id, status

Tyre
  tyre_id, manufacturer, model, size, rated_tkph, install_date,
  initial_tread_mm, cost, status

TyreFitment
  fitment_id, tyre_id, truck_id, position, installed_at,
  removed_at, installation_hours, removal_reason

SensorReading
  reading_id, timestamp, truck_id, tyre_id, position,
  pressure_kpa, tyre_temp_c, ambient_temp_c, payload_t,
  speed_kmh, gps_lat, gps_lon, imu_ax, imu_ay, imu_az,
  gyro_x, gyro_y, gyro_z, source, quality_flag

OperationWindow
  window_id, start_time, end_time, truck_id, route_id,
  distance_km, mean_payload_t, mean_speed_kmh, ambient_temp_c,
  idle_seconds, loaded_seconds, empty_seconds

TKPHRecord
  timestamp, tyre_id, mean_tyre_load_t, awss_kmh, tkph,
  rated_tkph, exceedance_ratio, exceedance_minutes, method_version

DamageInspection
  inspection_id, timestamp, tyre_id, truck_id, image_id,
  damage_present, damage_type, location, severity,
  confidence, model_version, reviewer_status

ImpactEvent
  event_id, timestamp, truck_id, tyre_id_or_null,
  gps_lat, gps_lon, peak_accel_g, jerk, duration_ms,
  event_type, severity, road_segment_id

Maintenance
  maintenance_id, tyre_id, timestamp, action, reason,
  labour_cost, parts_cost, downtime_hours, tread_after_mm

Failure
  failure_id, tyre_id, timestamp, failure_mode, cause,
  downtime_hours, replacement_cost, confirmed_by

### 1.2 Time-series key

Sensor rows are append-only. reading_id is unique, but the analytical key is (tyre_id, timestamp). If the TPMS sensor cannot identify a tyre directly, the gateway maps sensor serial number to fitment_id; never join only by truck because tyre position changes.

### 1.3 Event envelope

{
  "event_type": "sensor_reading",
  "schema_version": "1.0",
  "timestamp_utc": "2026-08-15T10:15:20Z",
  "truck_id": "DUMPER_07",
  "tyre_id": "TYRE_07_RR_OUT",
  "position": "rear_right_outer",
  "payload": {"pressure_kpa": 735, "tyre_temp_c": 82},
  "quality": {"source": "simulator", "confidence": 1.0}
}

The same envelope is used for real and synthetic data; source distinguishes simulator, sensor, manual_inspection, inferred, and operator_confirmed.

## 2. Sensor and IIoT mechanism

The practical architecture is:

TPMS / IMU / GPS / payload-CAN → edge gateway → MQTT → backend API → time-series + relational DB → analytics → dashboard.

Mining IIoT research emphasizes interoperability and the difficulty caused by fragmented vendors, communication, data-management and security infrastructure[6].

### 2.1 Recommended prototype hardware

- Pressure/temperature: real TPMS sensor if available; otherwise a laptop/Raspberry Pi simulator. Internal sensors are preferable because the objective is actual chamber pressure and temperature; commercial mining systems use internal readings and integrate with fleet systems[3].

- IMU: MPU-6050/ICM-20948 for the demonstrator; sample at 50–100 Hz. A mine-vehicle study showed that a small number of robust accelerometer/gyroscope sensors can reconstruct stress signals and estimate accumulated damage, while speed influence remains an unresolved issue[7].

- GPS: smartphone GNSS or USB GNSS at 1–10 Hz. Use map matching only after the raw track is stored.

- Payload/speed: simulator initially; real deployment should read the dumper’s authorised CAN/J1939 or fleet API rather than splice into safety-critical controls.

- Edge: Raspberry Pi 5 or industrial PC for gateway; ESP32 only for low-rate sensor acquisition.

- Network: BLE/RS-485 locally, Wi-Fi in a controlled demo, 4G/5G for mine backhaul. Use LoRaWAN only for low-rate sparse data, not raw IMU streams.

- Protocol: MQTT with QoS 1, retained device configuration, local buffering and replay after link loss.

### 2.2 MQTT topics

mine/{mine_id}/truck/{truck_id}/tyre/{tyre_id}/telemetry
mine/{mine_id}/truck/{truck_id}/imu
mine/{mine_id}/truck/{truck_id}/gps
mine/{mine_id}/tyre/{tyre_id}/inspection
mine/{mine_id}/alerts

### 2.3 Minimum edge processing

At the edge: timestamp synchronization, unit conversion, range checks, missing-value flags, IMU filtering, impact-event extraction and local pressure-loss alarms. Send both raw summary and derived event; do not discard raw values needed for later audit.

## 3. TKPH engine

### 3.1 Standard mechanism

TKPH is a workload/thermal proxy based on tyre load and average work-shift speed:

TKPH=Qmean×AWSS

where:

- Qmean = mean load carried by one tyre in tonnes;

- AWSS = average work-shift speed in km/h.

For a simple axle model:

Qmean,tyre=payload+tare_load_on_axlenumber_of_tyres_on_axle

Use the actual load distribution if the dumper exposes axle or wheel loads. If not, apply a documented axle distribution assumption and mark it as inferred.

### 3.2 Rolling/dynamic TKPH

For each rolling window of length W:

mean_payload = mean(payload[t-W:t])
mean_speed_loaded = distance_loaded / loaded_time
mean_tyre_load = load_distribution(mean_payload, truck_config)
tkph_window = mean_tyre_load * mean_speed_loaded

Do not average zero-speed samples into AWSS. Separate moving, loaded-moving, empty-moving and idle states. Store both:

- tkph_current: rolling 5–15-minute value;

- tkph_shift: shift/cycle summary;

- tkph_exceedance: cumulative time above rating.

Define:

exceedance_ratio=TKPHcurrentTKPHrated

and:

exceedance_minutes=t​ITKPHt>TKPHratedΔt

The site literature reports increasing real-site TKPH with average speed and ambient temperature, and nonlinear behaviour with cycle length[8]. OTR thermal modelling also uses vertical load, hauling speed and ambient temperature[9].

### 3.3 Decision use

- ratio < 0.80: normal workload.

- 0.80–1.00: monitor.

- 1.00–1.15: warning; inspect temperature trajectory.

- >1.15: high workload; recommend speed/load review.

These thresholds are prototype defaults, not manufacturer limits. Replace them with the tyre maker’s rated/design TKPH and mine engineering rules.

TKPH should influence risk indirectly: it raises the thermal-risk input and maintenance priority, but it should not override an actual sensor alarm. A commercial TPMS source explicitly presents actual chamber temperature as more correlated with belt temperature than TKPH alone[3].

## 4. Temperature prediction mechanism

### 4.1 Why not FE in the 36-hour build?

The strongest mechanistic OTR work uses a rubber heat-generation formulation, heat conduction and boundary conditions within a finite-element thermal model[9]. It is useful for offline calibration, but needs tyre geometry, material parameters, heat-transfer coefficients and validation sensors. Use it as the scientific rationale, not the live prototype model.

### 4.2 First-order thermal model

Use one internal thermal state:

Tt+Δt=Tt+Δtα⋅L⋅v⋅fp+βv2−γTt−Tamb−δIidle

Variables:

- Tt: current predicted tyre temperature in °C;

- L: mean tyre load, tonnes;

- v: speed, km/h converted consistently in calibration;

- fp: pressure factor, e.g. fp=Pref/P, clipped to a safe range;

- Tamb: ambient temperature;

- Iidle: 1 during idling/cooling state, otherwise 0;

- α,β: heat-generation coefficients;

- γ: cooling coefficient;

- δ: idle cooling adjustment;

- Δt: sampling interval in hours or seconds consistently.

A more stable implementation is:

Tpred,next=Tamb+e−kΔtTpred,current−Tamb+HL,v,PΔt

where H is a calibrated heat-generation rate.

### 4.3 Calibration

With real sensor data, fit α,β,γ,δ using ridge regression on temperature increments:

ΔT/Δt=b0+b1Lv+b2v2+b3T−Tamb+b4Iidle+b5Pref/P

Use the first 70% of chronological data for fitting and the final 30% for testing. Never randomly split time-series rows.

### 4.4 Abnormal trajectory

Compute:

residualt=Tactual,t−Tpred,t

and a smoothed residual:

rt=EMAresidualt

Alert when one of the following occurs:

- actual temperature exceeds a hard engineering limit;

- temperature slope is too high for the current TKPH;

- residual remains positive for N consecutive readings;

- pressure is low and residual is high;

- the model predicts cooling but actual temperature continues rising.

The residual is a model-disagreement signal, not proof of internal belt damage. It can also indicate sensor bias, changed tyre construction, wrong load distribution or missing road/speed features.

## 5. Tyre wear mechanism

### 5.1 What is feasible

For the prototype, estimate a wear rate from two or more tread-depth observations:

wear_rate=treadprevious−treadcurrentoperating_hourscurrent−operating_hoursprevious

If distance is reliable, also calculate mm/1,000 km. Extrapolate only within the observed operating regime:

estimated_hours_to_limit=treadcurrent−treadlimitwear_rate

Label this wear_projection, not validated RUL.

### 5.2 Feature pipeline

inspection tread depth + tyre hours + position + payload + speed
+ TKPH + temperature excursions + pressure history + road score
→ aggregate by tyre and time window
→ calculate wear rate
→ regression/gradient boosting baseline
→ wear-rate band and projection

Use robust linear regression first. If at least 30–50 tyre-level labelled histories exist, compare gradient boosting. Use tyre ID as a grouped split to prevent leakage.

### 5.3 Why not ANFIS/RUL now?

Survival analysis can model time to an observed removal/failure event, but requires censoring information and longitudinal event histories. A mining study used proportional hazards and found ambient temperature affected reliability and mean residual life[1]. A recent mining tyre study reported strong tread-depth influence and a GRNN–firefly hybrid model, but its reported performance is site-specific and does not substitute for cross-site validation[2]. Use those as future benchmarks, not as a reason to deploy a black-box model without data.

## 6. Computer vision: image → damage/type/location/severity

### 6.1 Model comparison

| Method | Output | Data requirement | Prototype judgement |
| --- | --- | --- | --- |
| Classification/CNN | whole-image label | image-level labels | Fastest, but no location. Use as fallback. |
| Object detection/YOLO | boxes + class | boxes or converted annotations | Best main choice for 36 hours. |
| Semantic segmentation | pixel class | pixel masks | Useful for cut area, but expensive labels. |
| Instance segmentation/Mask R-CNN | separate object masks | masks | Stronger localisation; slower and more complex. |
| Faster R-CNN | boxes/classes | box labels | Accurate but heavier; unnecessary for demo. |
| Vision Transformer | classification/detection | larger data, compute | Not justified with a small mining dataset. |
| Anomaly detection | deviation from normal | mostly normal data | Useful when damage classes are unknown; harder to explain and validate. |
| Few-shot learning | new classes from few examples | carefully curated support set | Future option, not first implementation. |

Public image evidence is mostly passenger-car/bike domain. TyreNet contains 1,698 images, with 866 defective and 832 good images, collected at service stations and licensed CC BY 4.0[10]. A public project uses Mask R-CNN and separate models for defect, sidewall, tread and wear, but it is not evidence of mining-tyre generalisation[11]. Mining-specific work describes fender-mounted continuous imaging and hazards such as embedded objects, but the retrieved summary provides limited reproducible benchmark detail[12].

### 6.2 Chosen mechanism

Use YOLO transfer learning for detection plus a small severity classifier.

image
→ crop/resize/normalise
→ quality check (blur, darkness, tyre visible)
→ YOLO: damage box + class + location
→ ROI crop
→ severity classifier/rule layer
→ inspection record + confidence + recommendation

Classes:

damage_type = cut | crack | puncture | embedded_object | tear | abrasion | unknown
location = tread | shoulder | sidewall | bead | unknown
severity = minor | moderate | severe

If the existing images only say normal versus damaged, do not invent type/severity labels. Train the binary model and create staged, manually labelled images for the additional taxonomy.

### 6.3 Severity without cut-depth measurement

Use ordinal visual features:

- Minor: small localised discontinuity; no visible structural exposure; no obvious deformation; no pressure loss.

- Moderate: longer/deeper-looking crack or cut, repeated branching, visible embedded object, shoulder/sidewall involvement, or persistent defect across inspections.

- Severe: large open tear, exposed reinforcement, bead damage, major sidewall deformation, rapidly increasing area, or damage accompanied by pressure loss/thermal abnormality.

These are operational labels, not millimetres. Start with a human-defined rubric and have two reviewers label a calibration set. For the demo, combine model confidence with image-derived features such as box area fraction, elongation, edge length, contrast, location and repeated detection. Do not claim structural depth or failure probability from RGB alone.

## 7. Fuzzy failure-risk engine

### 7.1 Why Mamdani is the correct prototype choice

Use a small Mamdani fuzzy inference system rather than ANFIS. A Mamdani system works with expert rules, is transparent, does not require failure labels, and supports explanations. ANFIS learns membership/rule parameters from labelled outcomes; without real tyre failures it will overfit synthetic labels and create false precision. Fuzzy predictive-maintenance studies show the general pattern: historical/knowledge-based inputs are mapped to linguistic variables and defuzzified into failure likelihood[13].

### 7.2 Inputs

Keep the first version to six risk dimensions:

- thermal_stress: temperature level + temperature slope + positive residual;

- pressure_stress: pressure deviation + loss rate;

- tkph_stress: current TKPH/rated TKPH + exceedance minutes;

- damage_stress: severity and location;

- wear_stress: wear projection / tread remaining;

- impact_stress: recent exposure-normalized impact burden.

Payload, speed, tyre age and operating history enter through these features. This avoids an unmanageable 10-input rule base.

### 7.3 Membership functions

Normalize each variable to 0–100 or 0–1. Use triangular/trapezoidal functions:

pressure_stress: normal (0–25), low (15–60), critical (50–100)
thermal_stress: normal (0–30), elevated (20–70), critical (60–100)
tkph_stress: normal (0–40), high (30–80), excessive (70–100)
damage_stress: none (0), minor (10–35), moderate (30–70), severe (65–100)
wear_stress: low (0–40), medium (30–75), high (65–100)
impact_stress: low (0–35), medium (25–70), high (60–100)
output risk: low (0–35), medium (25–70), high (60–100)

### 7.4 Rules

Use monotonic safety rules, for example:

IF pressure_stress is critical THEN risk is high
IF thermal_stress is critical THEN risk is high
IF damage_stress is severe THEN risk is high
IF damage_stress is moderate AND pressure_stress is elevated THEN risk is high
IF tkph_stress is excessive AND thermal_stress is elevated THEN risk is high
IF wear_stress is high AND thermal_stress is elevated THEN risk is high
IF impact_stress is high AND damage_stress is moderate THEN risk is high
IF two or more of {thermal, pressure, TKPH} are elevated THEN risk is medium/high
IF all stresses are normal AND no damage THEN risk is low

### 7.5 Inference and explanation

- Fuzzification: convert numeric values to membership degrees.

- Rule firing: calculate AND as minimum or product.

- Aggregation: take maximum contribution for each output class.

- Defuzzification: centroid of aggregated output membership function.

- Decision: score 0–100 plus label.

- Explanation: return the top three firing rules and raw values.

{
  "risk_score": 82,
  "risk_label": "HIGH",
  "reasons": [
    "TKPH is 1.18 times the rated value",
    "temperature is rising despite reduced speed",
    "moderate sidewall cut plus pressure loss"
  ],
  "recommended_action": "stop-and-inspect before next loaded cycle"
}

The score is not a calibrated probability unless validated against confirmed failures. The dashboard must display risk_score, risk_label, reasons, and data_quality separately.

## 8. Geographical damage-hotspot engine

### 8.1 Event detection

From IMU:

- remove gravity using orientation estimate or high-pass filtering;

- calculate resultant acceleration;

- detect peaks above threshold with refractory period;

- calculate peak g, jerk, duration and speed;

- attach nearest GPS point;

- assign event to road segment.

A mine-vehicle study used accelerometer/gyroscope data, coherence/LASSO signal selection, ARX stress estimation and rainflow damage accumulation; it also used accumulated damage to estimate road condition, while noting speed effects still needed work[7].

### 8.2 Map matching

For the hackathon, create a mine-road GeoJSON with segment IDs. Map each event to the nearest segment within a tolerance. Store both raw coordinates and assigned segment.

### 8.3 Exposure-normalized scoring

Raw event counts are biased toward heavily travelled roads. For segment s:

impact_rates=impact_eventsstruck-kms×100

damage_rates=confirmed_damagestruck-kms×100

failure_rates=confirmed_failuresstruck-kms×100

Use truck-km as the primary denominator because it measures exposure. Use events/trip as a secondary view when distance is unreliable. Use empirical-Bayes shrinkage or a minimum exposure threshold so one event on a rarely travelled segment does not become the top hotspot.

A defensible composite is:

Hs=0.35Zimpact_rates+0.40Zdamage_rates+0.25Zfailure_rates

where Z values are standardized within the mine. If confirmed damage/failure labels are sparse, show separate layers instead of pretending they are equally reliable.

### 8.4 Distinguish hotspot types

- Impact hotspot: high exposure-normalized IMU event rate; no damage outcome required.

- Damage hotspot: high confirmed image/inspection damage rate per truck-km.

- Failure hotspot: high confirmed removal/failure rate per truck-km.

A road can be an impact hotspot but not a failure hotspot. A thermal tyre hotspot is different: it is a tyre-state event, not a geographic cluster.

### 8.5 Algorithms

Start with road-segment scoring. Add DBSCAN only to discover unsegmented clusters. KDE is useful for smooth visual density but still needs exposure correction. Getis-Ord Gi* and spatial autocorrelation are future additions requiring enough events and stable spatial units. Transport hotspot research similarly shows that exposure-normalized/distribution-aware methods outperform frequency-only baselines[14].

## 9. End-to-end pipeline

raw sensor/image data
  → validate units, timestamps, IDs, quality
  → store immutable raw events
  → create 1-second/10-second telemetry windows
  → calculate load, speed, pressure-loss and temperature trends
  → calculate current/shift TKPH and exceedance
  → predict temperature and residual
  → calculate wear rate from tread observations
  → run image detector and severity layer
  → detect/map IMU impact events
  → aggregate exposure-normalized hotspot metrics
  → execute fuzzy risk rules
  → rank maintenance priority
  → serve API and dashboard/mobile views

### 9.1 Maintenance priority

Use:

priority=risk_score×criticality×data_confidence

where criticality reflects wheel position/vehicle mission and data_confidence discounts missing, stale or simulated data. A severe image with high confidence and pressure loss should outrank a weak TKPH-only warning.

Suggested actions:

HIGH + pressure loss or severe damage → stop/inspect immediately
HIGH thermal only → reduce load/speed and inspect temperature system
MEDIUM → inspect at next maintenance window
LOW → continue monitoring

## 10. Chosen methods table

| Component | Chosen method | Why | Input | Output | Complexity |
| --- | --- | --- | --- | --- | --- |
| Ingestion | MQTT event stream with local buffer | simple, decoupled, live-demo friendly | JSON telemetry/events | validated event bus | Low |
| TKPH | rolling load × loaded AWSS | transparent and aligned with mining practice | payload, axle/load assumption, speed, rated TKPH | current/shift TKPH, exceedance | Low |
| Temperature | first-order heat/cooling model + residual | physically interpretable and calibratable | load, speed, pressure, ambient, idle, actual temp | predicted temp, residual, trend alert | Low–Medium |
| Wear | robust wear-rate regression | needs few tread observations and avoids false RUL precision | tread, hours/distance, position, exposure | mm/hour, projection band | Low |
| Vision | YOLO transfer learning + ordinal severity rubric | localises damage and is feasible in 36 hours | image | class, box, location, severity, confidence | Medium |
| Risk | Mamdani fuzzy inference | explainable and does not require failure labels | six normalized stress inputs | score, Low/Medium/High, reasons | Low–Medium |
| Hotspots | road segments + truck-km normalization | interpretable and corrects travel-frequency bias | GPS, IMU events, damage/failure labels, distance | segment rates, ranking, map layers | Medium |
| Database | PostgreSQL + TimescaleDB or SQLite fallback | relational history plus time-series queries | all canonical records | queryable digital history | Low–Medium |
| Dashboard | Streamlit for demo | fastest integrated UI and maps/charts | REST/query outputs | live fleet/tyre/map views | Low |

## 11. Recommended 36-hour stack

### Backend

- Python 3.11

- FastAPI for typed REST endpoints

- Pydantic for event validation

- PostgreSQL + TimescaleDB if hosted; SQLite for local fallback

- SQLAlchemy for schema and queries

- Pandas/NumPy for feature windows

- scikit-learn for regression and baseline models

- scikit-fuzzy for Mamdani inference

### Vision

- Ultralytics YOLO for transfer learning/inference

- OpenCV for preprocessing and image quality checks

- Pillow for file handling

- Label Studio or Roboflow for rapid annotation

### Frontend and map

- Streamlit: fastest hackathon dashboard

- Plotly: trend plots and risk explanations

- Folium/pydeck: mine-road heatmap and segment ranking

### Deployment

- Docker Compose: backend, database, MQTT broker and dashboard

- Mosquitto MQTT broker

- Local laptop/Raspberry Pi demo first; cloud only if required

The stack is chosen for integration speed and explainability. Do not add Kafka, Kubernetes, a feature store, a digital twin platform or a transformer model in the 36-hour build.

## 12. Dataset specification

### 12.1 Real data that can support a demo

| Dataset | Fields | Real/synthetic | Use |
| --- | --- | --- | --- |
| TyreNet | 1,698 images; defective/good; cracks/wear/anomalies; smartphone images; CC BY 4.0[10] | Real, passenger-car/bike domain | pretraining and pipeline validation only |
| Public tyre-damage repositories | image files and labels vary; one Mask R-CNN project includes defect/sidewall/tread/wear models[11] | Real, domain mixed | baseline vision demo |
| Mine-vehicle research data | accelerometer, gyroscope, strain, speed and field-test signals in one published study[7] | Real research data, not assumed downloadable | mechanism precedent for IMU/road condition |
| Commercial system descriptions | pressure/temperature/GPS/analytics capabilities, not raw downloadable records[3][4] | Real product evidence | architecture requirements, not model training |

### 12.2 Synthetic data required now

telemetry.csv:
 timestamp, truck_id, tyre_id, position, payload_t, speed_kmh,
 ambient_temp_c, pressure_kpa, tyre_temp_c, lat, lon,
 ax_g, ay_g, az_g, tread_mm, source

inspection.csv:
 image_id, tyre_id, timestamp, damage_present,
 damage_type, location, severity, reviewer_label, source

failure.csv:
 tyre_id, timestamp, failure_mode, confirmed, source

Generate scenarios, not random noise:

- normal loaded cycle;

- gradual pressure leak;

- high payload/high speed/TKPH exceedance;

- hot ambient and slow cooling;

- impact cluster on one road segment;

- moderate sidewall cut followed by pressure loss;

- worn tyre with rising wear rate;

- sensor dropout and outlier.

The simulator must inject ground-truth scenario_id so every output can be tested.

### 12.3 Data required from CIL for real deployment

- truck and tyre master data;

- tyre sensor serial-to-position mapping;

- pressure/temperature at known sampling rate;

- payload and axle distribution;

- speed, distance, loaded/empty/idle state;

- ambient temperature and weather;

- GPS and IMU;

- tread measurements with timestamp and method;

- image inspection records with consistent camera geometry;

- tyre removal/failure date and confirmed cause;

- maintenance, rotation, repair and cost records;

- route/road-segment geometry and truck-km exposure.

No public source retrieved here establishes a row-level, mining-specific dataset containing all these fields, tyre IDs and failure outcomes. Treat such data as a deployment prerequisite, not a hidden assumption.

## 13. Module interfaces for three parallel developers

### Developer A — telemetry and analytics

Owns: simulator, MQTT, ingestion, database, TKPH, temperature, wear.

TelemetryWindow = {
  "tyre_id": str,
  "window_start": datetime,
  "window_end": datetime,
  "mean_payload_t": float,
  "mean_speed_kmh": float,
  "mean_pressure_kpa": float,
  "mean_temp_c": float,
  "ambient_temp_c": float,
  "distance_km": float
}

DerivedTelemetry = {
  "tyre_id": str,
  "tkph_current": float,
  "tkph_rated": float,
  "tkph_exceedance_ratio": float,
  "pressure_loss_kpa_per_h": float,
  "temp_slope_c_per_h": float,
  "temp_pred_c": float,
  "temp_residual_c": float,
  "wear_rate_mm_per_h": float | None
}

Tests: unit-test TKPH with hand calculations; simulate known heat input and verify temperature rises/cools; test tyre ID joins and missing data.

### Developer B — vision

Owns: image ingestion, labels, training/inference, inspection API.

DamageResult = {
  "image_id": str,
  "tyre_id": str | None,
  "damage_present": bool,
  "damage_type": str,
  "location": str,
  "severity": str,
  "confidence": float,
  "bbox": [float, float, float, float] | None,
  "model_version": str,
  "source": str
}

Tests: normal images should not create damage alerts; staged severe images should produce severe; low-light/blurred images should return quality_warning rather than confident classification.

### Developer C — geospatial, fuzzy risk and UI

Owns: IMU events, map matching, segment scoring, fuzzy rules, maintenance priority, dashboard.

HotspotResult = {
  "road_segment_id": str,
  "truck_km": float,
  "impact_events": int,
  "damage_events": int,
  "failure_events": int,
  "impact_rate_per_100_truck_km": float,
  "damage_rate_per_100_truck_km": float,
  "failure_rate_per_100_truck_km": float,
  "hotspot_score": float,
  "hotspot_type": str
}

RiskResult = {
  "tyre_id": str,
  "timestamp": datetime,
  "risk_score": float,
  "risk_label": str,
  "reasons": list[str],
  "recommended_action": str,
  "data_confidence": float
}

Tests: one simulated impact route must create one expected hotspot; doubling truck traffic without new events must lower the normalized rate; a severe cut plus pressure loss must produce High risk; all-normal input must produce Low risk.

### Integration rule

Each service reads/writes versioned JSON schemas through REST or a shared database. No service imports another developer’s internal Python modules. Integration happens only through:

POST /telemetry
POST /inspections
POST /impact-events
GET  /tyres/{tyre_id}/risk
GET  /trucks/{truck_id}/live
GET  /hotspots?start=&end=
GET  /maintenance/priorities

## 14. Hour-by-hour build plan

| Time | Deliverable |
| --- | --- |
| 0–3 h | Freeze schema, event envelope, repository structure and synthetic scenarios. |
| 3–8 h | Build simulator, MQTT/REST ingestion, database and live telemetry table. |
| 8–12 h | Implement TKPH, pressure-loss, temperature trend and first-order predictor. |
| 12–17 h | Implement fuzzy risk engine and reason generation. |
| 17–22 h | Implement GPS/IMU event detector, road segments and exposure-normalized hotspots. |
| 22–28 h | Train/integrate YOLO baseline; create normal/damaged and staged severity test set. |
| 28–33 h | Integrate dashboard, tyre history, risk cards, map and maintenance queue. |
| 33–36 h | Run scenario tests, fix interfaces, capture latency/accuracy results, document simulated versus real fields. |

## Final implementation choice

The build should use: rolling TKPH + first-order temperature state model + robust wear-rate regression + YOLO transfer learning with ordinal severity + road-segment hotspot rates normalized by truck-km + Mamdani fuzzy risk. This combination is simpler than FE thermal simulation, deep survival models, ANFIS or vision transformers, but preserves the mechanisms that matter: workload, thermal state, pressure, visible damage, cumulative wear, impact exposure and tyre history. The product’s strongest scientific claim should be that it provides explainable, multimodal maintenance prioritization; its weakest claims—validated failure probability, precise cut depth and time-to-failure—should remain future research until CIL supplies longitudinal tyre-level outcomes.

# References

1. 	Rahimdel MJ (2022) Residual lifetime estimation for the mining truck tires. Proceedings of the Institution of mechanical engineers Part D, journal of automobile engineering. https://doi.org/10.1177/09544070221121855

2. 	Kunkyin-Saadaari F, Kesseh A, Agadzie VK (2025) GRNN-FA hybrid model for predicting the useful life of dump truck tyres. Journal of Sustainable Mining. https://doi.org/10.46873/2300-3960.1451

3. 	TyreSense - Industrial Tire Pressure Monitoring System (TPMS) - Rimex. https://www.rimex.com/tyresense/

4. 	MICHELIN MEMS4: Premium mining TMPS solution. https://pro.africa.michelin.com/en/services/michelin-mems4

5. 	TOMS: A Proactive Tire Management System - Kal Tire Mining Tire Group. https://www.kaltiremining.com/en/tire-management-services/toms/

6. 	Aziz A, Schelén O, Bodin U (2020) A Study on Industrial IoT for the Mining Industry: Synthesized Architecture and Open Research Directions. IoT. https://doi.org/10.3390/iot1020029

7. 	Jakobsson E, Frisk E, Pettersson R, Krysander M (2017) Data driven modeling and estimation of accumulated damage in mining vehicles using on-board sensors. Annual Conference of the PHM Society. https://doi.org/10.36001/phmconf.2017.v9i1.2309

8. 	Ma S, Fan C, Liu W (2023) Effects of site operating conditions on real site TKPH (tonne-kilometer-per-hour) of ultra-large off-the-road tires. Proceedings of the Institution of mechanical engineers Part D, journal of automobile engineering. https://doi.org/10.1177/09544070231166166

9. 	Ma S, Wu L, Liu W (2022) Numerical investigation of temperatures in ultra-large off-the-road tires under operating conditions at mine sites. Journal of Thermal Science and Engineering Apllications. https://doi.org/10.1115/1.4056086

10. 	TyreNet: A High-quality Annotated Dataset for Tyre Defect ... https://data.mendeley.com/datasets/32b5vfj6tc

11. 	smu-ivpl/TireDamageDetection. https://github.com/smu-ivpl/TireDamageDetection

12. 	Silva GJF, Solorzano LFC, Suarez JT (2025) Real-Time Non-Intrusive Monitoring of Wear and Damage on Mining Truck Tires Using Digital Image Processing. International Journal of Electronics and Communication Engineering. https://doi.org/10.14445/23488549/ijece-v12i3p113

13. 	Camacho KSI, Roque MLP, Vicerra RRP, Bugtai NT (2025) Predictive Maintenance Algorithm for Machine Failure Using Fuzzy Logic Techniques. Proceedings of International Exchange and Innovation Conference on Engineering &amp; Sciences (IEICES). https://doi.org/10.5109/7395694

14. 	Qu W, Liu S, Zhao Q, Qi Y (2020) Methods for Identifying Truck Crash Hotspots. Journal of Advanced Transportation. https://doi.org/10.1155/2020/1751350